Offline SOP for Atom Charger

- Fault Name:Offline failure

- Fault code:F00001

- Fault phenomenon:Devices offline on platform

- Cause of failure:Communication module bracket defect

- Applicable models:Atom (WIFI version)

- Troubleshooting steps:

| procedure | Contents | Example (picture or reference file) |
| --- | --- | --- |
| Step1 | Disassembling the charging pile panel and pulling out the pins of the communication module; |  |
| Step2 | Replace the three metal screw brackets that hold the communication module, reinstall them, and confirm that the pin contacts are in place; |  |
| Step3 | Install all remaining accessories. |  |
| Step4 | Power up again and check again whether the registered monitoring platform is online; (The registered monitoring platform may be the monitoring platform owned by the customer or the official monitoring platform of Livoltek) If online again, the offline fault is resolved; If you are still offline, please contact your local after-sales team for support. | App Interface from Operator Part |