Low-volt Battery Short-circuit Treatment SOP

- Fault Name:

- Battery Short-circuit

- Battery Voltage Abnormal

- Fault code:

- BA1 0080, BA3 0080

- F1 0100000

- Fault phenomenon:The battery indicator light is red, and the inverter gives a battery short-circuit alarm

- Cause of failure:Inverter battery port short circuit

- Applicable models:BLF 51-5

- Troubleshooting steps:

| procedure | Contents | Example (picture or reference file) |
| --- | --- | --- |
| Step1 | Ensure that the positive and negative poles of the battery are not short-circuited; If short-circuited, please contact the local after-sales service team. |  |
| Step2 | When the battery is not short-circuited, turn off the battery air Switch, the DC Switch of the inverter, and the mains Switch, and remove the battery power line. |  |
| Step3 | Turn on the battery open again and observe the color of the battery indicator light. If it is green, the battery is fine,The short-circuit side is determined at the inverterThe battery port of. (Verification method: Use a multimeter, adjust it to buzz, and measure the Bat + and BAT-of the battery port of the inverter with a red and black probe; If there is a beeping sound, it proves that the inverter battery port is short-circuited; If there is no beep, please contact your local after-sales service team) If the indicator light is red or not illuminated, please contact your local after-sales service team. |  |